UNIVERSITY OF BRIDGEPORT

CS/CpE 460

Introduction to Robotics

Department of Computer Science and Engineering

Orientation.h

#include <iostream>

#include <iomanip>

#include <string>

#include <cmath>

#include <conio.h>

using namespace std;

class orientation

{

double a[10][4][4];

int n;

double p[4][4];

public:

orientation();

~orientation();

void m_input();

void m_output();

void t_output();

void initialize(double c[4][4]);

void operation(int);

void kinemetics(double m1[4][4],double m2[4][4]);

void puma();

};

orientation::orientation()

{

int k,i,j;

for(k=0;k<10;k++)

for (i =0;i<4;i++)

for (j =0;j<4;j++)

a[k][i][j]=0;

n=0;

}

orientation::~orientation()

{     int i,j;

for (i=0;i<4;i++)

for(j=0;j<4;j++)

p[i][j]=0;

}

void orientation::initialize(double c[4][4])

{

int i,j,k;

for(i=0;i<4;i++)

for(j=0;j<4;j++)

if(i==j)

c[i][j]=1;

else

c[i][j]=0;

for(k=0;k<n;k++)

for(i=0;i<4;i++)

for(j=0;j<4;j++)

a[k][i][j]=0;

}

void orientation::operation(int size)

{   int k,i,j;

double b[4][4],c[4][4];

n=size;

initialize(c);

if(n==6)

puma();

else

m_input();

for(k=0;k<n;k++)

{

for (i =0;i<4;i++)

for (j =0;j<4;j++)

b[i][j] = a[k][i][j];

kinemetics(c,b);

for (i =0;i<4;i++)

for (j =0;j<4;j++)

c[i][j]=p[i][j];

}

}

void orientation::kinemetics (double m1[4][4],double m2[4][4])

{

int i,j,k=0;

double sum;

while(k<4)

{

for(i=0;i<4;i++)

{

sum=0;

for(j=0;j<4;j++)

sum=sum + m1[i][j]*m2[j][k];

p[i][k]=sum;

}

k++;

}

}

void orientation::m_input()

{

int i,k;

const double M_PI = 3.141592;

double v[10][4];

string symbol="&a@d";

cout<<"\nEnter the "<<n<<" matrix(s)\n";

for(k=0;k<n;k++)

{

cout<<"Enter the A"<<k+1<<" Parameters\n";

for(i=0;i<4;i++)

{

cout<<symbol[i]<<k+1<<": ";

cin>>v[k][i];

cout<<"\n";

}

}

for(k=0;k<n;k++)

{

a[k][0][0]= cos (v[k][2]* (M_PI/180.0f));

a[k][0][1]= -1 * cos (v[k][0]* (M_PI/180.0f)) * sin (v[k][2]* (M_PI/180.0f));

a[k][0][2]= sin (v[k][0]* (M_PI/180.0f)) * sin (v[k][2]* (M_PI/180.0f));

a[k][0][3]= v[k][1] * cos (v[k][2]* (M_PI/180.0f));

a[k][1][0]= sin (v[k][2]* (M_PI/180.0f));

a[k][1][1]= cos (v[k][2]* (M_PI/180.0f)) * cos (v[k][0]* (M_PI/180.0f));

a[k][1][2]= -1 * sin (v[k][0]* (M_PI/180.0f)) * cos(v[k][2]* (M_PI/180.0f));

a[k][1][3]= v[k][1] * sin (v[k][2]* (M_PI/180.0f));

a[k][2][0]= 0;

a[k][2][1]= sin (v[k][0]* (M_PI/180.0f));

a[k][2][2]= cos (v[k][0]* (M_PI/180.0f));

a[k][2][3]= v[k][3];

a[k][3][0]= 0;

a[k][3][1]= 0;

a[k][3][2]= 0;

a[k][3][3]= 1;

}

}

void orientation::m_output()

{

int i,j,k;

cout<<"\n\n Your joint matrix(s) are:\n";

for(k=0;k<n;k++)

{

cout<<"\nA"<<k+1<<" is:\n\n";

for (i =0;i<4;i++)

{

for (j=0;j<4;j++)

{

cout.precision(2);

cout<<setw(4)<<a[k][i][j]<<"     ";

}

cout<<"\n";

}

}

}

void orientation::t_output()

{

int i,j;

system("cls");

cout<<"\n the T(0,"<<n<<") Matrix is: \n\n";

for(i=0;i<4;i++)

{

for(j=0;j<4;j++)

{

cout.precision(3);

cout<<setw(4)<<p[i][j]<<"     ";

}

cout<<"\n";

}

cout<<"\n the position of the end effector is:\n";

cout<<"X="<<p[0][3]<<"\n";

cout<<"Y="<<p[1][3]<<"\n";

cout<<"Z="<<p[2][3]<<"\n";

cout<<"\n the orientation of the end effector is:\n";

for(i=0;i<3;i++)

{

for(j=0;j<3;j++)

cout<<setw(4)<<p[i][j]<<"     ";

cout<<"\n";

}

getch();

}

void orientation::puma()

{

int k;

const double M_PI = 3.141592;

double v[10][4];

cout<<"\n ITS A Puma 560 Robot values of &,a, and d of A1,A2,..,A6 already uploaded\n";

cout<<"Enter @1,@2,...,@6 Values in degrees\n";

for(k=0;k<6;k++)

{

cout<<"\nEnter @"<<k+1<<": ";

cin>>v[k][2];

}

v[0][1]=v[0][3]=v[1][0]=v[1][3]=v[3][1]=v[4][1]=v[4][3]=v[5][0]=v[5][1]=v[5][3]=0;

v[0][0]=v[3][0]=90;

v[2][0]=v[4][0]=-90;

v[1][1]=0.431800;

v[2][1]=0.020300;

v[2][3]=0.150050;

v[3][3]=0.431800;

for(k=0;k<6;k++)

{

a[k][0][0]= cos (v[k][2]* (M_PI/180.0f));

a[k][0][1]= -1 * cos (v[k][0]* (M_PI/180.0f)) * sin (v[k][2]* (M_PI/180.0f));

a[k][0][2]= sin (v[k][0]* (M_PI/180.0f)) * sin (v[k][2]* (M_PI/180.0f));

a[k][0][3]= v[k][1] * cos (v[k][2]* (M_PI/180.0f));

a[k][1][0]= sin (v[k][2]* (M_PI/180.0f));

a[k][1][1]= cos (v[k][2]* (M_PI/180.0f)) * cos (v[k][0]* (M_PI/180.0f));

a[k][1][2]= -1 * sin (v[k][0]* (M_PI/180.0f)) * cos(v[k][2]* (M_PI/180.0f));

a[k][1][3]= v[k][1] * sin (v[k][2]* (M_PI/180.0f));

a[k][2][0]= 0;

a[k][2][1]= sin (v[k][0]* (M_PI/180.0f));

a[k][2][2]= cos (v[k][0]* (M_PI/180.0f));

a[k][2][3]= v[k][3];

a[k][3][0]= 0;

a[k][3][1]= 0;

a[k][3][2]= 0;

a[k][3][3]= 1;

}

}

Multiply.cpp

#include <iostream>

#include <iomanip>

#include <string>

#include <cmath>

#include <conio.h>

#include "orientation.h"

using namespace std;

void main()

{

int n=0,flag=0,f=0;

orientation obj;

while ( f!=5)

{

system("cls");

cout<<"\n\n\n\nPlease enter your choice\n";

cout<<"1-Determine the size (6 for PUMA 560)\n";

cout<<"2-Enter DH parameters\n";

cout<<"3-Display the Matrices\n";

cout<<"4-Result T\n";

cout<<"5-exit\n\n\n";

cin>>f;

if(f==1) {

system("cls");

cout<<"enter n\n";

cin>>n;

flag=0;

}

if(f==2)   {

system("cls");

if(n)

{

flag=1;

obj.operation(n);

}

}

if((f==3)&&flag)

{system("cls");

obj.m_output();

getch();

}

if(f==4)

{

if(n&&flag)

obj.t_output();

}

}

}

Output:

Main Screen:

Entering DH parameters for A1 and A2. Size was set as 2

Displaying A1 and A2:

Result T(0,2), Position and orientation of the end effector:

Puma 560 entries:

The Matrices A1,A2,……,A6