University of Bridgeport

CS/CpE 460 Introduction to Robotics

Fall 2011

Assignment 2: Connecting the SRV-1 to Computer using a serial to USB connector.

Table of Contents

Table of Contents	3

Introduction	4

Preparing the Connection	4

Required Hardware	4

Steps	4

Connect the Robot to the Computer	6

Required Software	6

Steps	6

Uploading C code to the Robot	10

Requirements	10

Steps	10

References	12

# Introduction

The object of this assignment is to compile C code that makes use of the camera and upload it to the Surveyor's SRV-1 onboard memory. To do this, first we need to connect the robot to a computer using USB connection. Then download the robot firmware, modify it to add the extra code, and finally upload the code to the machine.

# Preparing the Connection

## Required Hardware

- Surveyor SRV-1 robot.

- Pololu CP2102 USB-to-Serial Bridge (CP2102 chip).

- USB converter cable (Mini-B to standard).

- Computer running Windows 7.

- Soldering kit, Wires, Screw driver, and bins.

## Steps

- To the each end of three wires, solder a bin.

- Solder the wires to the CP2102 chip connecting to the following bins:

  - TX

  - RX

  - GND

- Make sure that all the wires are connected firmly to the chip and mark each wire with the chip equivalent bin name.

- Using the screwdriver, remove the camera and attached chip from the robot top.

- Connect the wires bins to the robot using the following configuration: If this is looking at the back of the robot, consider the bins numbered as the following:

16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

32 31 30 29 28 27 26 25 24 23 22 21 20 19 18 17

Then connect the wires as:

TX = 02 
GND = 17 
RX = 18

- Connect the CP2102 chip to the USB cable.

# Connect the Robot to the Computer

## Required Software

- Silabs 210x Windows XP/Vista/7 signed driver.

- Surveyor Br@y terminal.

## Steps

- Download and install the CP2102 driver from https://www.silabs.com/Support%20Documents/Software/CP210x_VCP_Win2K_XP_S2K3.zip

- After installing the program, the final step is to scan the system for the connected chip, Click yes.

- Wait until the installation is successful

- Connect the USB cable to the computer; a dialog will appear trying to install the new hardware.

- If Windows was unable to install the new hardware automatically then follow these steps:

  - Open the “Device Manager” and look for “Pololu USB01B USB to Serial Adapter” and open its properties.

  - Go the driver tab and click “Update Driver” the select “Browse my computer for a software driver” after that select "Let me pick from a list of drivers on my computer".

  - Go to control panel and click “Add a device” from the “Hardware and Sound” category and wait for the program to find the new device (May take 10+ minutes).

  - Select "Show All Devices" then click "Next" and for the program to populate all the options and devices.

  - Once all the fields are populated; from the manufacture list select “Silicon Laboratories” and from the model list select “Silicon Labs CP210x USB to UART Bridge” and click Next.

  - You will get a message indicating that the driver is not verified, click Yes to install anyway.

  - If the driver is installed correctly, the CP2102 red led will light on and you will get a success message.

- Download Br@y terminal from http://www.surveyor.com/srvdownload/termv19b.zip and extract the terminal executable file.

- Now the robot is connected and ready.

# Uploading C code to the Robot

## Requirements

- The CP2102 is installed and connected to the computer.

- The CP2102 driver is installed and ready to be used.

- Br@y terminal application.

## Steps

- Connect the USB cable to the CP2102 and the computer.

- Run the Br@y terminal.

- From the “COM” section, select “COM 3”.

- Click “connect”, if all the above steps were successful the CP2102 red led will light on and stay lit.

- To test if the terminal can read the robot, turn the robot switch ON then OFF, if the received section showed <00> then the robot is ready for file transfer.

- Download the robot firmware to modify it and upload it again. There is a choice of running interpreted C or BASIC onboard, in this assignment we decided to use C.

  - Download the SRV-1 controller firmware from http://www.surveyor.com/srvdownload/srv1.hex.053007c  and then rename the file to srv1.hex.

  - Download the Python console  from http://www.surveyor.com/srvdownload/pySRV1Console-022807.zip

  - Use the Python version of SRV1Console to edit your C or BASIC programs stored in onboard flash memory.

- We used a simple C program for this assignment. The program is supposed to run the motor and set right to 50 and left to 50 (100 is max), capture a picture frame, then enable video feed, then delay (do nothing) for 500 milisecond, then turn the motors off (stop moving) as shown below:

main()

{

motors(50, 50);

vcap();

vcam(7);

delay(500);

vcam(0);

motors(0, 0);

}

- Modify the firmware and include the above code, then using Br@y click send file, select the compiled file and upload to the robot.

# References

- http://www.surveyor.com/C.html

- http://www.surveyor.com/cgi-bin/yabb2/YaBB.pl?num=1200515001

- http://www.societyofrobots.com/robotforum/index.php?topic=9821.0

- http://www.robotreviews.com/reviews/surveyor-srv-1-blackfin-robot-review

- http://forum.pololu.com/viewtopic.php?f=3&t=2271

- http://www.pololu.com/catalog/product/391

- http://www.pololu.com/docs/0J7

- http://www.surveyor.com/srvdownload/termv19b.zip

- http://www.chiark.greenend.org.uk/~sgtatham/putty/download.html

- http://www.surveyor.com/cgi-bin/yabb2/YaBB.pl?num=1215912392

- http://www.surveyor.com/cgi-bin/yabb2/YaBB.pl?num=1196352263

- http://www.voxatec.com/downloads/RoboBrain_Manual.pdf